Open1722 CAN Latency Tester

Linux vs. Zephyr IEEE-1722 Gateway

Project Specification Document

# 1. Team

| Name | Role | Date | Signature |
| --- | --- | --- | --- |
| Sripiranavan Yogarajah | Author / Developer |  |  |
| Sebastian Schildt | Client |  |  |
| Ulrich Straus | Client |  |  |

# 2. Motivation

In modern automotive and industrial systems, CAN data often must be transported over Ethernet backbones using time-sensitive networking protocols. The IEEE-1722 (AVTP) standard defines how to encapsulate CAN frames into Ethernet packets. Open1722 is an open-source implementation of IEEE-1722 that runs on both Linux and Zephyr OS, making it an ideal candidate for comparison.

Every gateway introduces latency and — critically — jitter, which can destabilise time-sensitive control loops. This project builds a measurement framework to quantify and compare the end-to-end latency and jitter of CAN forwarding through an IEEE-1722 bridge on both a Linux host (Raspberry Pi) and a Zephyr (STM32F439ZI) node, providing a real-world performance delta between a general-purpose OS and an RTOS.

# 3. Goals

- Implement a low-jitter CAN source on RPi 1 (Linux) that generates CAN frames with predictable timing using timerfd_create() and SCHED_FIFO.

- Deploy Open1722 on both Raspberry Pi nodes (Linux) to bridge CAN over Ethernet via IEEE-1722, and compare with a Zephyr-based implementation on the STM32F439ZI.

- Measure and compare end-to-end round-trip latency and jitter of the forwarding path for both implementations.

- Identify the dominant sources of jitter in each implementation (OS scheduling, driver buffering, network stack).

- Use the STM32F439ZI as the latency measurement node: it sends frames on CAN1, receives them back on CAN2, and computes latency = t3 - t0 entirely on-device.

# 4. Hardware Setup

The following hardware components are used in this project:

| Component | Model / Detail | Role |
| --- | --- | --- |
| Microcontroller | STM32F439ZI (Nucleo-144) | CAN sender (t0) + receiver (t3), latency measurement |
| CAN Transceiver x2 | SN65HVD230 | CAN bus physical layer, 3.3V compatible |
| Single-board Computer x2 | Raspberry Pi 4 | RPi1: CAN source + IEEE-1722 sender; RPi2: IEEE-1722 receiver + CAN sink |
| CAN HAT x2 | Raspberry Pi CAN HAT | CAN interface for each Raspberry Pi |
| Network | Direct Ethernet cable | Point-to-point between RPi1 and RPi2 (no switch, lower jitter) |

# 5. System Architecture

## 5.1 Overview

The system uses a five-node round-trip pipeline. The STM32F439ZI acts as both the CAN frame source and the latency measurement node. Two Raspberry Pi units act as IEEE-1722 gateways. The pipeline is:

[STM32 CAN1] → [Transceiver 1] → [CAN Bus A] → [RPi1 CAN HAT]

→ [RPi1 Open1722] → [Ethernet IEEE-1722] → [RPi2 Open1722]

→ [RPi2 CAN HAT] → [CAN Bus B] → [Transceiver 2] → [STM32 CAN2]

## 5.2 STM32F439ZI Pin Assignment

| Interface | TX Pin | RX Pin | Connector |
| --- | --- | --- | --- |
| CAN1 | PD1 | PD0 | CN9 pin 8 (TX) / pin 10 (RX) |
| CAN2 | PB6 | PB5 | CN10 pin 13 (TX) / CN7 pin 13 (RX) |
| 3.3V power | — | — | CN8 pin 7 |
| GND | — | — | CN7 pin 8 |

Note: PB5 (CAN2 RX) shares CN7 pin 13 with PA7 by default. Solder bridge SB35 must be ON and SB33 OFF to route PB5 to this connector pin.

## 5.3 Timestamp and Latency Measurement

Latency is measured entirely on the STM32F439ZI to avoid the need for clock synchronisation between nodes:

| Timestamp | Location | Event |
| --- | --- | --- |
| t0 | STM32 — CAN1 TX | STM32 sends CAN frame on CAN1 (PD0/PD1) |
| t1 | RPi1 — CAN HAT RX | RPi1 receives frame on CAN Bus A, encapsulates into IEEE-1722, sends over Ethernet |
| t2 | RPi2 — CAN HAT TX | RPi2 receives IEEE-1722 packet, decapsulates, sends frame on CAN Bus B |
| t3 | STM32 — CAN2 RX | STM32 receives frame on CAN2 (PB5/PB6) |
| Result | Round-trip latency = t3 − t0 (measured on STM32, no external clock sync needed) |  |

# 6. Requirements

### CAN Source

- Low-jitter CAN transmitter running on RPi1 using timerfd_create() with a SCHED_FIFO thread and CPU pinning.

- Generates periodic frames at a defined bitrate (500 kbps).

- A sequence counter is embedded in the payload for reordering and loss detection.

### IEEE-1722 Gateway — Linux (RPi1 + RPi2)

- RPi1: receives CAN frame from CAN Bus A via CAN HAT, encapsulates into IEEE-1722 AVTP, sends over Ethernet.

- RPi2: receives IEEE-1722 packet over Ethernet, decapsulates, forwards CAN frame onto CAN Bus B via CAN HAT.

- Uses Open1722 Linux-native POSIX socket backend with SocketCAN (can0).

### IEEE-1722 Gateway — Zephyr (STM32F439ZI)

- Receives CAN via interrupt-driven callback, pushes into k_msgq.

- Forwarding thread encapsulates frame via Open1722 into an IEEE-1722 PDU and sends over Ethernet.

- Implements the same forwarding function as the Linux gateway for direct comparison.

### Measurement & Diagnostics

- STM32 records t0 and t3 timestamps using Zephyr's k_cycle_get_32() or hardware timer.

- Per-packet latency, mean, min, max, and standard deviation reported over UART shell.

- CAN bus speed: 250 kbps (configurable).

- 120 ohm termination resistors at both ends of each CAN bus segment.

# 7. Solution Approach

The system implements a five-node round-trip pipeline as described in Section 5. The key design decisions are:

- Direct Ethernet cable between RPi1 and RPi2 (no switch) to minimise network jitter and isolate the Open1722 forwarding latency from switch-induced variability.

- STM32F439ZI as the exclusive measurement node — by computing latency entirely on-device (t3 - t0), no PTP or external clock synchronisation is required.

- CAN1 on STM32 runs in loopback mode in software (Zephyr CAN_MODE_LOOPBACK) when not connected to a bus, as CAN2 on STM32F4 requires CAN1 to be clocked.

- Both Linux and Zephyr Open1722 gateways are tested under identical bus conditions to produce a fair comparison of determinism.

- Jitter is considered the primary metric of interest, as it reveals scheduling non-determinism more clearly than mean latency alone.

# 8. Roadmap

| Task | Start | End | Status |
| --- | --- | --- | --- |
| Hardware Setup & CAN Bus Verification | March 30, 2026 | March 31, 2026 | Done |
| CAN TX/RX on Linux (SocketCAN) and Zephyr | April 1, 2026 | April 3, 2026 | Done |
| Low-Jitter CAN Source Implementation | April 4, 2026 | April 7, 2026 | In Progress |
| Open1722 Integration — Linux Gateway (RPi1 + RPi2) | April 8, 2026 | April 12, 2026 | Pending |
| Open1722 Integration — Zephyr Gateway (STM32) | April 13, 2026 | April 20, 2026 | Pending |
| Review 1: Basic IEEE-1722 Forwarding (Both Nodes) | April 21, 2026 | April 22, 2026 | Milestone |
| Timestamp & Latency Measurement Framework | April 23, 2026 | May 5, 2026 | Pending |
| Jitter Analysis & Source Optimization | May 6, 2026 | May 15, 2026 | Pending |
| Stress Testing (High Bus Load, Burst Traffic) | May 16, 2026 | May 25, 2026 | Pending |
| Review 2: Data Analysis & Linux vs. Zephyr Comparison | May 26, 2026 | May 27, 2026 | Milestone |
| Final Documentation & Presentation | May 28, 2026 | June 15, 2026 | Pending |

# 9. Impediments and Measures

| Jitter from Linux Scheduler | Risk: Linux is not an RTOS; thread preemption causes unpredictable delays in the forwarding path, making fair comparison difficult. |
| --- | --- |
|  | Measure: Use SCHED_FIFO with mlockall() on Linux to minimise scheduling jitter. Use chrt and CPU pinning (taskset) to isolate the forwarding thread on RPi1 and RPi2. |

| Clock Sync for Cross-Node Timestamps | Risk: Without a common clock, comparing TX timestamps (source) with RX timestamps (sink) on different nodes is inaccurate. |
| --- | --- |
|  | Measure: The round-trip measurement approach avoids this entirely: both t0 and t3 are recorded on the STM32F439ZI using its local clock. No external time reference or PTP is required. |

| Open1722 Zephyr Port Maturity | Risk: The Zephyr port of Open1722 may be less mature than the Linux version, introducing additional bugs or missing features. |
| --- | --- |
|  | Measure: Start with the Linux port on both RPis to validate the full pipeline end-to-end first, then port to Zephyr incrementally using the same test vectors. |

| CAN Bus Congestion / Overrun | Risk: At high bus load, the gateway cannot clear the RX FIFO fast enough, causing dropped frames. |
| --- | --- |
|  | Measure: Use CAN hardware filtering and Zephyr's k_msgq for deep buffering. Monitor TEC/REC counters via UART shell. Use sequence numbers in payload to detect drops. |